# MATERIAL SAFETY DATA SHEET (MSDS)

SECTION 1	PRODUCT AND COMPANY IDENTIFICATION

(IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY/UNDERTAKING)

### 1.1. Product identifier

Product Description: MFE 7516 VINYL ESTER (STYRENE-FREE)

### Chemical family:

| 1.2. Intended Use Sector of Uses [SU] Uses advised against | Environmental friendly corrosion resistance resin SU3 - Industrial uses SU12 - Manufacture of plastics products, including compounding and conversion SU22 - Professional uses No information available |
| --- | --- |

### Details of the supplier of the safety data sheet Manufacturer information:

Lateral Repairs UAB

Address: Paberžių g. 5, Tauragė, LT-72328, Lithuania Mail: Info@lateralrepairs.com

Phone: +37061212882

Office: +37069876581

### Emergency telephone number

Emergency telephone +37061212882, or contact your local emergency telephone number

SECTION 2	HAZARDS IDENTIFICATION

### 2.1 Classification of the substance or mixture Classification according to Regulation (EC) No. 1272/2008

Skin irritation, H315	Category 2

Eye irritation, H319	Category 2

Skin sensitization, H317	Category 1

For the full text of the H-Statements mentioned in this Section, see Section 16.

### Classification according to EU Directives 67/548/EEC or 1999/45/EC

R43

Xi Irritant R36/38

For the full text of the R-phrases mentioned in this Section, see Section 16.

2.2 Label elements

### Labelling according to Regulation (EC) 1272/2008 (CLP) Pictogram

### Signal word: Warning Hazard statement(s)

H315 Causes skin irritation.

H317 May cause an allergic skin reaction. H319 Causes serious eye irritation.

### Precautionary statement(s)

P280 Wear protective gloves.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

### Supplemental Hazard Statements

None

SECTION 3	COMPOSITION/INFORMATION ON INGREDIENTS

Not a hazardous substance or mixture according to Regulation (EC) No 1272/2008 For the full text of the R phrases mentioned in this Section, see Section 16

For the full text of the H-Statements mentioned in this Section, see Section 16

SECTION 4	FIRST-AID MEASURES

### General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

### If inhaled

Remove victim to fresh air and keep at rest in a position comfortable for breathing. Get medical advice/attention if you feel unwell.

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

### In case of skin contact

Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a physician.

### In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

### If swallowed

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

Indication of any immediate medical attention and special treatment needed no data available

SECTION 5	FIRE FIGHTING MEASURES

### Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

### Special hazards arising from the substance or mixture

Carbon oxides, nitrogen oxides (NOx)

### Advice for firefighters

Wear self-contained breathing apparatus for firefighting if necessary.

### Further information

Use water spray to cool unopened containers.

SECTION 6	ACCIDENTAL RELEASE MEASURES

### Personal precautions, protective equipment and emergency procedures

Wear respiratory protection. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel to safe areas. Beware of vapors accumulating to form explosive concentrations. Vapors can accumulate in low areas.

### Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

### Methods and materials for containment and cleaning up

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13). Keep in suitable, closed containers for disposal.

### Reference to other sections

For disposal see section 13.

SECTION 7	HANDLING AND STORAGE

### Precautions for Safe Handling

Avoid contact with skin and eyes. Avoid inhalation of vapor or mist.

Keep away from sources of ignition - No smoking. Take measures to prevent the build up of electrostatic charge.

### Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Recommended storage temperature: no more than 25 °C Moisture sensitive. Light sensitive.

### Specific end use(s)

Exposure scenario	No information available

Other Guidelines	No information available

SECTION 8	EXPOSURE CONTROL/PPE (PERSONAL PROTECTION EQUIPMENTS)

### Exposure limits

Contains no substances with occupational exposure limit values.

### Biological occupational exposure limits

8.2. Exposure controls

Engineering Controls:	Avoid contact with skin, eyes and clothing. Wash hands before breaks and immediately after handling the product.

### Personal protective equipment Eye/face protection

Face shield and safety glasses Use equipment for eye protection tested and approved under

appropriate government standards such as NIOSH (US) or EN 166(EU).

### Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

### Body Protection

Complete suit protecting against chemicals. The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace. Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

SECTION 9	PHYSICAL/CHEMICAL PROPERTIES

### Information on basic physical and chemical properties

| - Appearance Form Color - Odor - Odor Threshold - pH - Melting point/freezing point - Initial boiling point and boiling range - Flash point - Evaporation rate - Flammability (solid, gas) - Upper/lower flammability/explosive limits - Vapor pressure - Vapor density - Relative density - Water solubility - Partition coefficient: octanol/water - Auto-ignition temperature - Decomposition temperature - Viscosity - Explosive properties - Oxidizing properties | clear, liquid yellow transparent liquid ester-like no data available no data available < -60 °C 67 °C at 4,7 hPa-lit. 96 °C - closed cup no data available no data available no data available 0,1 hPa at 20 °C no data available 1.1-1.20 g/mL at 25 °C no data available log Pow: -0,53 at 20 °C no data available no data available no data available no data available no data available |
| --- | --- |

## OTHER INFORMATION

### No information available

SECTION 10	STABILITY AND REACTIVITY

### Reactivity

no data available Chemical stability no data available

### Contains the following stabilizer(s):

Mequinol ((>=180 - <=220 ppm) Possibility of hazardous reactions no data available

### Conditions to avoid

Exposure to light. May polymerize on exposure to light.

### Incompatible materials

Strong acids, Strong bases, Strong oxidizing agents, Strong reducing agents

### Hazardous decomposition products

Other decomposition products - no data available

SECTION 11	TOXICOLOGICAL INFORMATION

### 11.1. Information on toxicological effects Chemical name:

2-Hydroxyethyl methacrylate (CAS #: 868-77-9)

### Acute toxicity

LD50 Oral - rat - 5050 mg/kg

LD50 Dermal - rabbit - > 3.000 mg/kg

### Skin corrosion/irritation

Skin - rabbit

Result: irritating - 24 h

Serious eye damage/eye irritation Eyes - rabbit

Result: Moderate eye irritation - 24 h (Draize Test)

### Respiratory or skin sensitization

Maximization Test - guinea pig

May cause sensitization by skin contact. (OECD Test Guideline 406)

Germ cell mutagenicity no data available Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

### Reproductive toxicity

Reproductive toxicity - rat - female - Oral

Effects on Fertility: Pre-implantation mortality (e.g., reduction in number of implants per female; total number of implants per corpora lutea). Effects on Embryo or Fetus: Fetotoxicity (except death, e.g., stunted fetus). Effects on Embryo or Fetus: Fetal death.

### Specific target organ toxicity - single exposure

no data available

### Specific target organ toxicity – repeated exposure

no data available

### Aspiration hazard

no data available Additional Information RTECS: Not available

To the best of our knowledge, the chemical, physical, and toxicological properties have not been

thoroughly investigated.

SECTION 12	ECOLOGICAL INFORMATION

Toxicity to fish flow-through test - Pimephales promelas (fathead minnow) - 227 mg/l - 96 h

### Persistence and degradability

Biodegradability aerobic Chemical oxygen demand - Exposure time 28 d Result: 84 % - Readily biodegradable.

(Closed Bottle test) Bioaccumulative potential no data available

### Mobility in soil

no data available

### Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

### Other adverse effects

no data available

SECTION 13	DISPOSAL CONSIDERATIONS

### Waste treatment methods

This combustible material may be burned in a chemical incinerator equipped with an afterburner and scrubber. Offer surplus and non-recyclable solutions to a licensed disposal company.

### Contaminated packaging

Dispose of as unused product.

SECTION 14	TRANSPORT INFORMATION

### UN number

ADR/RID: - IMDG: - IATA: -

UN proper shipping name ADR/RID: Not dangerous goods IMDG: Not dangerous goods IATA: Not dangerous goods Transport hazard class(es) ADR/RID: - IMDG: - IATA: -

### Packaging group

ADR/RID: - IMDG: - IATA: -

### Environmental hazards

ADR/RID: no IMDG Marine Pollutant: no IATA: no

### Special precautions for user

no data available

SECTION 15	REGULATORY INFORMATION

Safety, health and environmental regulations/legislation specific for the substance or mixture Labelling according to Regulation (EC) No 1272/2008 Hazard pictograms Please refer section 2. Signal word Please refer section 2.

Hazard-determining components of labeling: Please refer section 2.

Hazard statements Please refer section 2. Precautionary statements Please refer section 2. Chemical safety assessment Please refer section 2.

Chemical safety assessment: A Chemical Safety Assessment has not been carried out.

### √ EINECS - European Inventory of Existing Commercial Chemical Substances (EINECS) Note:

"√" Indicates that the substance included in the regulations

SECTION 16	OTHER INFORMATION

Full text of H-Statements referred to under sections 2 and 3. Eye Irrit. Eye irritation

H315 Causes skin irritation.

H317 May cause an allergic skin reaction. H319 Causes serious eye irritation.

Skin Irrit. Skin irritation

Skin Sens. Skin sensitization

Full text of R-phrases referred to under sections 2 and 3 Xi Irritant

R36/38 Irritating to eyes and skin.

R43 May cause sensitization by skin contact. Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. The information in this document is based on the present state of our knowledge and is applicable to the product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the product.